# Terms of reference and technical Specifications

- General information

| # Assignment name | # Supply, installation and commissioning of a professional cinematographic film scanner for the digitisation of analogue film materials (16 mm and 35 mm) |
| --- | --- |
| # Beneficiary | # Leading Committee for the Iraqi Visual Memory |
| # Country | # Iraq |

- Context and justification of the need

  - Context of the project

The Iraqi Cinematheque Project supported by the French Ministry for Europe and Foreign Affairs and implemented by Expertise France aims to safeguard and enhance the cinematic archives held by the Leading Committee for the Iraqi Visual Memory.

Years of crisis in Iraq have caused significant damage to the country, including its infrastructure and historical heritage. Some cinema and media archives have been preserved, but under very rudimentary conditions that have accelerated their deterioration. Yet, these archives represent a heritage of great historical and symbolic value. Since Iraq lacks both the necessary expertise in film archive restoration and digitization, as well as the required equipment, the project seeks to train Iraqi professionals in the management, restoration, and promotion of film and audiovisual archives. The project focuses on a collection of 104 Iraqi fiction films (shot on 16mm and 35mm film) dating from the 1940s to the 2000s.

- Component 1: Evaluation and assessment of the archive collection (a hundred emblematic fiction films from Iraqi cinema) and inventory of the collection.

- Component 2: Cleaning, repairing, digitizing, and digitally restoring the cinematic archives.

- Component 3: Dissemination and promotion of the restored and digitized film archives through film screenings in Baghdad and related events across Iraq.

  - Justification of the need

The implementation of the Component 2 of the Iraqi Cinematheque Project has started and the Iraqi team needs to start digitize the film restored manually. To proceed with this digitisation the provision of a professional film scanner is required. This technical annex defines the minimum technical specifications for the supply of a professional cinematographic film scanner intended for the digitisation, preservation and valorisation of audiovisual heritage, within the framework of the Iraqi Cinematheque project.

The equipment will be operated by a heritage institution and must be fully suitable for the handling and digitisation of old, fragile and degraded film materials.

- Detailed description of the supply and services

  - General Description of the Scanner System

The system shall include, at a minimum:

- A professional, frame-by-frame cinematographic film scanner for 35mm and 16mm.

- A dedicated graphic workstation for control and processing the scanning films.

- All software required for scanning and file output.

- All accessories necessary for normal operation for 35mm and 16mm.

- Associated services, including  delivery in Iraq, installation, commissioning and training

- Warranty and after-sales support for the film scanner for a minimum period of three (3) years, including maintenance coverage, spare parts availability, and technical assistance (remote/on-site) as required.

- The equipment supplied under this contract shall be brand new, unused, and of current production. Refurbished, second-hand, demonstration, or previously installed equipment shall not be accepted. All components shall be delivered in original manufacturer condition and shall benefit from the full manufacturer’s warranty as specified in this document.

    - Supported Film Formats

The scanner shall support the digitisation of the following film formats:

- 16 mm film (including Super 16)

- 35 mm film

The system must be suitable for:

- Positive and negative film

- Sound and silent film

- Old, fragile, shrunken or physically damaged film elements

    - Scanning Technology

The scanner shall meet the following requirements:

- Frame-by-frame scanning

- Sprocketless film transport system (capstan-based or equivalent)

- LED or laser illumination with low thermal emission

- High-resolution digital image sensor 3+ flashes

- Image stabilisation and flicker reduction

- Optical Sound Reader

- Fine adjustment controls (exposure, colour balance, gamma)

Continuous video telecine systems are not acceptable.

The ability to digitise materials with measurable shrinkage (typically up to 1.5–2%) and to support variable and non-standard frame rates (including 16, 18, and 24 fps) is particularly important in the context of the Iraqi Cinematheque collections.

    - Resolution and Image Quality

- Minimum resolution:

  - Native 2K for 16 mm

  - Native 4K for 35 mm

- Extended dynamic range (HDR or equivalent)

- Colour fidelity suitable for long-term archival preservation

    - Output Formats and Deliverables

The system shall allow output in the following formats:

- DPX (image sequence)

- TIFF

- ProRes or equivalent professional intermediate codec

- Associated technical metadata (timecode, scanning parameters)

Generated files must be compatible with professional restoration and long-term archival workflows.

    - Graphic Workstation and Software

The supply shall include:

- A professional-grade graphic workstation (or compatibility with such a workstation)

- Software for:

  - Scanner operation

  - Image preview

  - Acquisition settings

  - File export

- Software licences shall be perpetual or clearly defined

  - General Description of the Expected Services:  Delivery, Installation and Training

    - Transport and delivery

The equipment shall be shipped by airfreight and delivered to Baghdad International Airport.

Delivery shall be made under customs control at the airport, on a CIP (Carriage and Insurance Paid To) basis, in accordance with Incoterms® 2020, including the insurance coverage.

The successful bidder shall provide the following documents for the transport of the scanner:

- the Certificate of Origin,

- the HS Code (Harmonized System customs code of the equipment),

- a detailed packing list,

- the serial number of the equipment,

- the Air Waybill (AWB),

- the insurance policy covering the shipment.

The above-mentioned documents to be provided after contract award, not at the tender stage, in order to facilitate import customs clearance.

    - Initial calibration and commissioning

The equipment shall be delivered, installed, commissioned and handed over fully operational, ready for immediate use.

The supplier shall be responsible for supervising the installation and shall ensure the correct, stable and compliant operation of the scanner at the final site in Baghdad. This shall include, as a minimum, system configuration, initial calibration, testing and validation under real operating conditions at the installation location.

The supervision of installation, calibration and commissioning shall be carried out either:

- on-site in Baghdad by the supplier’s qualified personnel, or

- remotely, through appropriate technical means, provided that the supplier guarantees effective supervision and full operational conformity of the equipment.

In all cases, the supplier shall remain fully responsible for the proper installation, commissioning and performance of the scanner.

    - Training

The supplier shall provide training for the operators for a minimum duration of three (3) to five (5) days, ensuring that the Iraqi technical team is fully capable of operating the scanner independently.

The training shall be delivered through one or a combination of the following modalities:

- On-site training in Baghdad, at the installation location; and/or

- Online or remote training, for follow-up support and additional sessions as required.

The training arrangements shall be as follows:

- Either a qualified representative or engineer of the supplier shall be present in Baghdad to deliver the installation support and initial training;

- Or at least one member of the Iraqi technical team shall receive hands-on training at another location, on a machine of similar technical specifications (e.g. at the supplier’s premises or at an agreed facility) with a qualified representative or engineer of the supplier.

In the latter case, all costs related to the training delivered outside Iraq, including travel, accommodation and subsistence, shall be borne entirely by the supplier.

The supplier shall include in the offer the detailed program of the training.

    - Warranty and maintenance

The equipment shall be covered by a minimum warranty period of thirty-six (36) months from the date of final acceptance.

The supplier shall guarantee the availability of spare parts for a minimum period of ten (10) years following the delivery of the equipment.

Upon request of the Contracting Authority or its designated partner, the supplier shall be able to supply and ship spare parts within reasonable timeframes, under conditions to be agreed.

The Contractor shall provide a list of recommended spare parts and consumables for preventive maintenance and shall include an initial starter kit of essential spare parts necessary for at least two (2) years of normal operation.

The supplier shall provide remote technical support during the warranty period and thereafter, including troubleshooting and operational assistance.

An optional maintenance contract may be proposed by the supplier and shall be detailed separately in the financial and technical offer.

  - Documents to be provided

The tenderer shall provide, as part of the technical offer, the following documents:

Technical documentation in English, including:

- User manuals;

- Maintenance guides;

- Safety procedures;

- Workflow and operational documentation;

- Warranty conditions, clearly detailing the scope, duration and applicable terms;

- Certificate of conformity of the equipment, demonstrating compliance with applicable technical and safety standards;

- Technical data sheet indicating the weight and dimensions (volume) of the machine, in order to allow technical and logistical comparison between offers.

- Schedule of the service and associated deliverables

  - Schedule

Desired start date of the contract: April 27, 2026

Expected date of arrival of the scanner in Baghdad July 27, 2026

Expected date for the training of the Iraqi staff: from August 3 to 7, 2026

  - Anticipated deliverables

| Deliverables | End date |
| --- | --- |
| - Preparation of the scanner Brief report attesting that the professional film scanner is ready to be shipped to Iraq | T0 + 2 month |
| - Shipment of the scanner Report attesting that professional film scanner has been delivered to Iraq | T0 + 3 months |
| - Installation of the scanner and training of the staff Detailed report of the installation of the scanner in Bagdad and the training of the staff | T0 + 4 months |
| - Final report Final file compiling: -The 3 above-mentioned reports -The document attesting the approval of the equipment -The evaluation by the Iraqi team of the training conducted on the use of the scanner by the supplier | T0 + 5 months |

  - Coordination

The service provider shall designate a single contact person for project implementation purposes to be the sole contact person for Expertise France.

A launch meeting shall be held right after the contract award has been notified.

Close collaboration must take place from assignment preparation right up to completion. Furthermore, regular exchanges must take place on assignment progress and any difficulties that may be encountered.

- Required expertise by the supplier

  - Profile of the Company

The tenderer shall be a company with proven experience in the supply and integration of professional cinematographic film scanning systems intended for archival, heritage or institutional use.

The company must demonstrate:

- At least 5 years of experience in the field of cinematographic digitisation, film scanning or audiovisual archiving solutions;

- Proven experience in supplying professional-grade film scanners suitable for the digitisation of 16 mm and 35 mm film;

- References from cinematheques, national archives, audiovisual heritage institutions, laboratories or comparable organisations;

- Capacity to provide international delivery, on-site installation and commissioning, including in complex or remote environments;

- Ability to provide after-sales service, technical support and maintenance, either directly or through authorised partners.

The tenderer shall provide a brief company profile and a list of relevant references demonstrating its experience in similar projects.

  - Profile and Qualifications of the Trainer

The tenderer shall designate a qualified professional responsible for the on-site training of Iraqi technical teams.

The proposed trainer must meet the following minimum requirements:

- At least 8 years of professional experience in cinematographic film digitisation and/or audiovisual archiving workflows;

- Demonstrated hands-on experience in the operation of professional film scanners (16 mm and 35 mm);

- Strong knowledge of:

  - Film handling and preservation principles;

  - Scanning workflows and quality control;

  - File formats and archival standards (DPX, TIFF, ProRes or equivalent);

- Proven experience in delivering technical training to operators or archivists, preferably in an institutional or international context;

- Ability to deliver training in English, using clear pedagogical methods adapted to the local context, capacity to express him/herself in Arabic would be an asset.

The training shall cover both theoretical and practical aspects, including:

- Safe handling of film materials;

- Scanner operation and calibration;

- Workflow management and file output;

- Basic troubleshooting and preventive maintenance.

The tenderer shall submit the CV of the proposed trainer as part of the technical offer.